PPD EZ Li-Po base plate instructions Clean bottom of battery and back side of plate with degreaser / cleaner prior to mounting. Place weight into chassis then slide battery on top of plate. Align battery to clear chassis and choose location, remember, the further from center of chassis the weight, the less weight that will be required. Using fine tip felt pen mark outside edge of battery onto plate and remove. Using thin servo tape, or for even tighter clearances industrial carpet tape (many track will have this) Note Best adhesion will occur at 70 degs. or higher. Lay 3 pieces onto plate at each end and center (humps bottoms, bond double thickness to battery between humps :-). Remove tape backing, align battery to pen marks being careful to keep parallel to plate, and press into place. Hold for 1 minute. Base complete! - now install side weight. Check back for product updates and plate options at ppbbillet.com ®

PPD EZ Li-Po side weight instructions Note— For best results place weight on outside edge of battery. Remember, the further from center of chassis the weight, the less weight that will be required. If using 3.0 oz base plate, bond plate to bottom of battery prior to mounting weight. Best adhesion will occur at 70 degs. or higher. Clean outside edge of battery and back side of plate with degreaser / cleaner prior to mounting. For universal weight, add weights and temporarily place on battery, then test balance prior to bonding. Place 3 strips of thin servo tape at each end and center after bonding optional weights into place. Remove tape backing, align bottom edge of weight flush to base plate or to bottom of battery and press secure. Hold for 1 minute. You are done…go have fun!! Check back for product updates and plate options at ppbbillet.com ®

PPD EZ Li-Po chassis inserts instructions. Clean bottom of battery and inserts prior to mounting. Tape one side of inserts with thin servo tape or for low clearance bulkheads uses industrial carpet tape (many tracks will carry this) Load battery into chassis and find optimum placement. Remember, the further from center of chassis the weight (or battery), the less weight to balance that will be required. Best adhesion will occur at 70 degs. or higher. Remove backing on 2 inserts and press and hold into 2 outside chassis slots. Now try to remove battery to see if there is enough clearance to run inserts. If there is too much difficulty in removing battery contact kjp@ppdbillet.com for return. If removal cleared , re-install battery and add remaining 4 inserts. Base complete! - now install side weight. Check back for product updates and plate options at ppbbillet.com ®